TT130SMC THERMAL TRANSFER PRINTER

Quick Start Guide

# Operations Overview

## Unpacking and Inspection

This printer has been specially packaged to withstand damage during shipping.  Please carefully inspect the packaging and printer upon receiving the bar code printer. Please retain the packaging materials in case you need to reship the printer.

Unpacking the printer, the following items are included in the carton.

- One printer unit

- One Windows USB Key Drive for Drivers / Manuals.

- One quick installation guide

- One power cord

- One auto switching power supply

- One USB interface cable

- One printer caddy

- One cardboard take up roll

If any parts are missing, please contact HellermannTyton Support: 1.800.537.1512

## Printer Overview

### Front View

4

3

1

2

5

1. Ribbon access cover

2. Top cover open lever

3. LED indicator

4. Feed button

5. SD card socket

### Interior View

### Rear View

4

3

2

1

5

1. Power switch

2. Power jack socket

3. USB interface

4. Ethernet switch

5. Fan-fold paper entrance chute

# Setup

## Setting Up the Printer

1. Place the printer on a flat, secure surface.

2. Make sure the power switch is off.

3. Plug the power cord into the AC power cord socket at the rear of the printer, and then plug the power cord into a properly grounded power outlet.

4. Connect the provided USB cable to the printer and then connect to your computer.

## Open the Top Cover

1. Open the printer top cover by pulling the tabs located on each side towards the front of the printer, then lift the top cover to the maximum open angle.

2. A top cover support at the right rear of the printer will engage with lower inner cover to hold the printer top cover open.

## Load the Ribbon

## (TT230SMC shown, SAME PROCESS for TT130SMC)

1. Open the printer’s top cover by pulling the top cover open levers located on each side of the printer and lifting the top cover to the maximum open angle.

2. Open the ribbon access cover.

Note:

1. In normal printing mode, ribbon access cover can be opened while opens the top cover. Ribbon access cover can be closed while top cover is open or close.

2. In peeler and cutter mode, please open the top cover then the ribbon access cover can be opened or closed.

3. Insert the paper core right side onto the rewind hub. Align the notches on the left side and mount onto the spokes.

4. Insert the ribbon right side onto the supply hub. Align the notches on the left side and mount onto the spokes.

5. Stick the ribbon onto the ribbon rewind paper core.

6. Turn the ribbon rewind gear until the ribbon plastic leader is thoroughly wound and the black section of the ribbon covers the print head. Then close the ribbon access cover and the top cover.

Loading Labels

- Separate Green Guides and lock to side

- Install labels on Caddy

- Feed labels as shown

- Adjust the label guides

(1)

(2)

(3)

- Adjust the label guides using the small gear to the right side. The label stock should fit between the two guides, but not too tightly.

# Stock should move easily between the guides.

# Correct Setup Shown Below

Close the Cover

Hold the top cover and press the top cover support (push inwards) to disengage the top cover support with lower inner cover; gently close the top cover.

Turn on power to the printer.

The light on the top of the printer, located by the “Feed” button should turn green.

# You’ve successfully completed the physical printer setup!

# Let’s install the printer drivers on the next page…

Installing the printer drivers

Plug your USB stick into a separate port on your computer; this stick contains your printer driver.

SELECT “Open folder to view files”

Access the files on the USB stick

DOUBLE CLICK ON 7.3.4 file TO OPEN THE DRIVER INSTALLER

May be slightly different depending on date of release

CLICK ON I accept and Next

Next

Click on Finish

Ensure that “Install printer drivers” is selected and click on NEXT.

Select the TT130SMC

Click on Next.

Select or highlight USB001 and click NEXT

The default printer name is TT130SMC

Click Next.

Click FINISH and your printer driver should be ready to use!